# Bittu Xerox Kiosk

# Technical Architecture Specification v2.0

Autonomous 24/7 Self-Service Printing & Document Kiosk for Indian University Campuses

Target: KL University, Vaddeswaram, Andhra Pradesh

# 1. Executive Summary

Bittu Xerox Kiosk is a compact, autonomous printing station designed to replace manual photocopy shops in university hostels. It operates 24/7 without human supervision, accepting documents via QR code upload, Telegram bot, or direct campus ERP integration, and delivers printed output at Rs 2.50 per page (52% gross margin).

The system runs BittuOS, a purpose-built Linux distribution based on Raspberry Pi OS Lite, featuring a read-only filesystem that survives power cuts, OTA remote updates, and fleet management capabilities for multi-campus scaling.

# 2. Problem Statement & Operational Reality

Curfew Bottleneck: 15,000+ students served by only 2 manual counters that close at 5:30 PM. Zero late-night printing access for hostel students.

Queue Starvation: Morning 8:00-9:00 AM assignment rushes create 30-45 minute queues, causing students to miss lectures.

Severe Data Privacy Leakage: Shopkeepers mandate WhatsApp file forwarding. Confidential Aadhaar cards, resumes, and exam papers remain on shared devices indefinitely.

Manual Payment Overhead: Cash change hassles, manual UPI screenshot verification, and lost student tabs create friction.

# 3. End-to-End System Architecture

The platform is architected around a decoupled, event-driven model consisting of four core tiers:

Ingress & User Tier: Telegram Bot (@BittuXeroxBot), WhatsApp API Adapter (Phase 2), QR Code Web Upload, and Chromium Touchscreen Kiosk PWA on 15.6" FHD display.

Core Application Backend: High-concurrency FastAPI microservice handling file validation, format conversion (LibreOffice headless), pricing calculation, dynamic UPI QR generation, and ERP integration.

Ephemeral Print Engine: CUPS daemon on Raspberry Pi 5 with 512MB volatile RAM disk (tmpfs) and post-print auto-deletion. Express Lane chunked preemption scheduler for fair queuing.

Hardware & IoT Tier: Brother HL-L2321D laser printer, 600VA UPS, HC-SR501 PIR sensor (3-tier power management), dual 120mm thermal fans, and Modbus RS485 energy telemetry.

# 4. BittuOS - Custom Kiosk Operating System

## 4.1 Why a Custom OS

When scaling from 1 kiosk to 50+, the operating system becomes the product. A general-purpose Raspberry Pi OS carries 400+ MB of unnecessary packages, exposes a desktop environment, and uses a writable filesystem vulnerable to SD card corruption during power cuts.

BittuOS strips Pi OS Lite to essentials, locks the filesystem to read-only, and boots the kiosk application automatically in 12 seconds.

## 4.2 Architecture

| Layer | Component | Purpose |
| --- | --- | --- |
| Kernel | Linux 6.6 (Pi 5 BSP) | Hardware abstraction, drivers |
| Init | systemd | Service management, boot orchestration |
| Filesystem | Read-only rootfs (overlayfs) | Immune to power cut corruption |
| Spool | tmpfs at /tmp/kiosk_spool (512MB) | Volatile RAM buffer for documents |
| Data | /data partition (ext4, rw) | SQLite DB, config, logs only |
| Display | Cage Wayland compositor | Minimal compositor for Chromium |
| Kiosk UI | Chromium in --kiosk mode | Fullscreen PWA on localhost:8000 |
| Backend | FastAPI + Uvicorn | Job management, payments, ERP |
| Bot | python-telegram-bot | Telegram ingress adapter |
| Print | CUPS + brlaser driver | Print job management |
| Power | PIR daemon (3-tier) | Sleep/wake management |
| Watchdog | bcm2835_wdog + systemd | Auto-reboot on hang (60s timeout) |
| Updates | bittu-updater daemon | OTA update agent with rollback |
| Monitoring | bittu-heartbeat daemon | Reports status every 5 minutes |

## 4.3 Boot Sequence (12 seconds)

0.0s - GPU firmware loads

1.5s - Linux kernel boots

3.5s - systemd initializes all services in parallel

5.0s - NetworkManager connects (WiFi or Ethernet)

5.5s - CUPS, FastAPI, Telegram bot start

7.0s - Cage Wayland compositor starts

9.0s - Chromium kiosk opens http://localhost:8000

12.0s - Ready for first user

Compare: Standard Raspberry Pi OS Desktop boots in 45-60 seconds. Windows (used by local shops) boots in 2-4 minutes.

## 4.4 Read-Only Filesystem

The root filesystem is mounted read-only using overlayfs. Writes are redirected to a tmpfs overlay that is discarded on reboot. This means:

Power cuts cannot corrupt the OS - the SD card is never written to during normal operation.

No apt updates can break the system - the package manager is disabled.

A fresh reboot always returns to a known-good state.

Persistent data (SQLite DB, config) lives on a separate /data partition.

## 4.5 OTA Update Mechanism

For scaling to multiple kiosks, each unit runs an update agent that checks a central server every 30 minutes:

1. Check /api/updates/latest for new version

2. Download update.tar.gz and verify SHA-256 hash

3. Extract to /opt/bittu-staging/

4. Atomic swap: rename live to rollback, staging to live

5. Restart services and run health check

6. If health check fails, automatic rollback to previous version

7. Report success/failure to central fleet dashboard

## 4.6 Fleet Management Dashboard

A central web dashboard (hosted on VPS) provides real-time visibility into all deployed kiosks:

Online/offline status with uptime tracking

Current software version per kiosk

Toner level, paper count, CPU temperature

Pages printed today, revenue generated

Error alerts (paper jam, payment failure, overheating)

Remote service restart capability via WireGuard VPN

## 4.7 Phased OS Evolution

| Phase | Kiosks | OS Approach | Image Size | Boot Time |
| --- | --- | --- | --- | --- |
| V1 (Pilot) | 1-5 | Hardened Pi OS Lite + overlayfs | ~800 MB | ~12 sec |
| V2 (Scale) | 10-50 | Buildroot custom build | ~80 MB | ~6 sec |
| V3 (Enterprise) | 50+ | Yocto (industrial-grade) | ~100 MB | ~6 sec |

# 5. Ingress Architecture: Telegram Bot First

The Bittu Kiosk implements a Telegram Bot First strategy with an abstracted messaging adapter interface:

Zero API Cost: 100% free API with zero per-message costs.

2GB File Ceiling: Supports large thesis PDFs and drawing files (WhatsApp caps at 100MB).

Rich Inline Keyboards: Students select Single/Duplex, Copies (1-20), Page Ranges, Color/Watermark, and University Logo Stamp directly in-chat.

4-Digit OTP PIN: Generated after configuration. Valid for 2 hours with 3 attempts/minute rate limiting.

WhatsApp Phase 2: Same bot logic, different transport adapter. Requires Meta Business verification and per-message fees.

# 6. Express Lane Chunked Preemption Scheduler

Standard FIFO print queues cause queue starvation: an 80-page lab manual blocks 10 students waiting for 1-page hall tickets.

Solution: Page-Boundary Preemptive Chunking Algorithm

1. Jobs <= 5 pages are classified as EXPRESS. All others are BULK.

2. Bulk jobs are split into 5-page micro-chunks.

3. After each chunk, the scheduler checks the Express queue.

4. If an Express job is waiting, it prints immediately (6 seconds for 1 page), then the bulk job resumes.

5. UX: Student B waits 15 seconds instead of 8 minutes. Student A barely notices the 6-second pause.

This is a marketing differentiator: "Express Print - small jobs never wait." No competitor offers this.

# 7. Campus ERP Integration

The kiosk integrates directly with the university student portal (e.g., KL ERP) through a secure adapter pattern:

1-Tap Exam Hall Tickets: Students authenticate via Registration Number + OTP, then print exam admit cards with dynamic security watermarks.

Grade Cards & Certificates: Semester grade cards, attendance reports, and fee receipts printed directly from ERP.

Secure Session Bridge: ERP tokens and student credentials are NEVER exposed to the kiosk frontend or stored in memory.

Phase 2 Feature: Requires registrar approval and authorized API access from KL University IT department.

# 8. University Logo Watermark Stamp

A unique value-add feature: students can add the university logo as a watermark or stamp on any document.

Implementation: PyPDF2/reportlab overlays the KLU logo on printed pages.

Use cases: Assignment cover pages, official submissions, faculty exam papers with anti-cheating watermarks.

Revenue: Optional +Rs 1/page premium for stamped documents.

# 9. Zero-Retention Security (DPDP Compliant)

Compliant with India's Digital Personal Data Protection Act, 2023:

Volatile RAM Buffer (tmpfs): Files exist strictly in RAM at /tmp/kiosk_spool. NEVER written to SD card storage.

Post-Print Auto-Delete: When CUPS emits job-completed signal, the file is immediately removed from RAM via os.remove().

TLS 1.3 In-Transit: All file transfers (QR upload, Telegram API) use end-to-end TLS encryption.

OTP Security: PINs stored in-memory with 2-hour TTL auto-expiry. Rate-limited to 3 attempts per minute. This rate-limiting (not hashing) is what prevents brute-force attacks on a 4-digit PIN space.

# 10. Smart Power Architecture: 3-Tier PIR Management

HC-SR501 PIR motion sensor drives a 3-tier power state daemon, achieving 71% energy savings:

| Tier | Trigger | Actions | Power Draw | Wake Time |
| --- | --- | --- | --- | --- |
| Active | PIR detects motion | Everything ON, full brightness | ~75W idle | - |
| Light Sleep | 3 min no motion | Display dims 30%, printer standby | ~22W | 3 sec |
| Deep Sleep | 15 min no motion | Display OFF, printer deep sleep | ~4W | 8 sec |
| Night Mode | 2:00-6:00 AM | Peripherals OFF via relay, Pi minimal | ~1.5W | 15 sec |

Monthly power cost drops from Rs 443 (always-on) to Rs 127 (with 3-tier management).

# 11. Hardware Bill of Materials (V1 Corrected)

| Category | Component | Model | Budget (Rs) |
| --- | --- | --- | --- |
| Core | Raspberry Pi | Pi 5, 2GB RAM | 8,000 |
| Core | Active Cooler | Pi 5 Active Cooler | 600 |
| Core | Power Supply | 27W USB-C PSU | 1,300 |
| Core | Storage | 64GB A2 MicroSD (Industrial) | 1,200 |
| UI | Touchscreen | 15.6" IPS HDMI Capacitive FHD | 9,000 |
| Printing | Laser Printer | Brother HL-L2321D | 12,200 |
| Printing | Toner | Compatible Brother toner | 1,500 |
| Presence | PIR Sensor x2 | HC-SR501 | 150 |
| Energy | Energy Meter | RS485 Modbus DIN-rail | 2,000 |
| Energy | RS485 Adapter | USB to RS485 | 300 |
| Connectivity | USB Hub | 4-port USB 3.0 | 400 |
| Controls | Push Buttons x2 | 22mm illuminated | 400 |
| Audio | Speaker + Buzzer | USB speaker + 5V buzzer | 600 |
| Power | UPS | 600VA line-interactive | 2,700 |
| Body | Enclosure | MDF + acrylic + fasteners + hinges | 4,400 |
| Power | Extension + Protection | Surge strip + MCB/fuse | 1,000 |
| Consumables | A4 Paper x3 | 75 GSM, 500-sheet reams | 1,050 |
| Misc | Cables | USB, HDMI, Ethernet | 1,000 |
|  |  | HARDWARE TOTAL | 47,800 |
| Marketing | Branding + Posters + Standee + Domain + QR + Coupons + Ambassadors |  | 6,100 |
|  |  | GRAND TOTAL | 53,900 |

# 12. Unit Economics

| Component | Cost/Page | Share |
| --- | --- | --- |
| Paper (75 GSM) | Rs 0.52 | 43% |
| Toner Cartridge | Rs 0.25 | 21% |
| Drum Amortization | Rs 0.10 | 8% |
| Electricity | Rs 0.03 | 3% |
| Jam/Misprint Buffer | Rs 0.05 | 4% |
| Campus Floor Share | Rs 0.25 | 21% |
| Total Production Cost | Rs 1.20 | 100% |
| Selling Price | Rs 2.50 |  |
| Gross Profit/Page | Rs 1.30 | 52% margin |

## 12.1 Payback Scenarios

| Scenario | Pages/Day | Monthly Profit | Payback |
| --- | --- | --- | --- |
| Conservative | 150 | Rs 4,875 | ~11 months |
| Realistic | 350 | Rs 11,375 | ~4.7 months |
| Peak Season | 700 | Rs 22,750 | ~2.4 months |

# 13. Technical Bottlenecks & Mitigations

## 13.1 Hardware Risks

| Bottleneck | Severity | Mitigation |
| --- | --- | --- |
| SD card corruption from power cuts | CRITICAL | Read-only rootfs (overlayfs) + industrial-grade SD card |
| Printer duty cycle exceeded (10K/month rated, 17.5K peak) | HIGH | Scheduled cool-down periods + budget for 18-month replacement |
| Thermal throttling in enclosed cabinet (40C+ ambient) | MEDIUM | Active cooler + dual 120mm fans + ventilation mesh |
| Paper jams with no clerk present | HIGH | Front-access jam clearing + on-screen instructions + Telegram alert |
| Physical vandalism in hostel corridors | HIGH | CRCA sheet metal (production), tamper-proof screws, CCTV positioning |

## 13.2 Software Risks

| Bottleneck | Severity | Mitigation |
| --- | --- | --- |
| File format diversity (.docx, .pptx, .jpg) | HIGH | LibreOffice headless for conversion + Pillow for images |
| Payment webhook delays (10-60 sec) | HIGH | Dual verification: webhook (push) + polling every 3 sec (pull) |
| CUPS/Brother driver compatibility | MEDIUM | Use open-source brlaser driver, pin version, never auto-update |
| Concurrent file uploads during peak | MEDIUM | Async job queue, sequential LibreOffice conversion |

## 13.3 Operational Risks

| Bottleneck | Severity | Mitigation |
| --- | --- | --- |
| Paper resupply (500 sheets lasts 1.4 days at peak) | HIGH | IR paper sensor + Telegram alerts + student worker (Rs 2K/month) |
| University bureaucracy (permissions, power, network) | HIGH | Ideathon win for legitimacy + internal channel via faculty |
| Peak exam load (50+ students in 2-hour window) | MEDIUM | Express Lane scheduler + queue position display + 2nd kiosk at scale |

# 14. Software Stack Summary

| Module | Technology | Purpose |
| --- | --- | --- |
| OS | BittuOS (hardened Pi OS Lite) | Read-only, 12-sec boot, OTA updates |
| Kiosk UI | Chromium PWA | 15.6" touchscreen interface |
| Backend | Python 3.11 + FastAPI | Async job management + APIs |
| Database | SQLite | Jobs, users, payments, logs |
| Printing | CUPS + brlaser | Print queue management |
| Scheduler | Express Lane algorithm | Page-boundary chunked preemption |
| Bot | python-telegram-bot | File upload, inline menus, OTP |
| Converter | LibreOffice headless | .docx/.pptx/.xlsx to PDF |
| Logo Stamp | PyPDF2 / reportlab | University watermark overlay |
| Payments | UPI QR + Razorpay webhook | Dynamic QR + dual verification |
| Power | PIR + systemd daemon | 3-tier sleep management |
| Monitoring | Heartbeat agent | Fleet dashboard reporting |
| Updates | bittu-updater daemon | OTA with automatic rollback |
| Security | iptables + tmpfs + TLS | Firewall, RAM spool, encryption |

# 15. 60-Day Pilot Execution Plan

| Phase | Days | Deliverables |
| --- | --- | --- |
| Hardware Procurement | 01-15 | Order all BOM items, collect from Vijayawada dealers, verify specs |
| Chassis Fabrication | 10-20 | MDF cutting (Auto Nagar), acrylic panels, 3D printed mounts, assembly |
| BittuOS Build | 15-25 | Strip Pi OS, install stack, configure services, enable read-only rootfs, create base image |
| Software Integration | 20-35 | FastAPI backend, Telegram bot, CUPS setup, Express Lane scheduler, payment gateway |
| Testing & Hardening | 35-45 | Load testing (500 pages), payment flow, jam recovery, power cut simulation, security audit |
| Campus Deployment | 45-50 | Install in KLU hostel lobby, electrical hookup, network config, student onboarding |
| Live Pilot | 50-60 | 10-day monitored operation, collect metrics, iterate on UX, prepare scale plan |

## 15.1 Pilot Definition of Done

A student can send a PDF or DOCX via Telegram, select print options via inline keyboard, receive a dynamic UPI QR code, pay via PhonePe/GPay, walk to the kiosk, enter a 4-digit PIN on the 15.6" touchscreen, and collect printed pages in under 60 seconds, with zero bytes of persistent document storage retained on the device.

# 16. Competitive Positioning

| Feature | Qwikprint PRO | Qwikprint MINI | Bittu Xerox Kiosk |
| --- | --- | --- | --- |
| CAPEX | Rs 3.49 Lakh | Rs 1.65 Lakh | Rs 53,900 |
| Revenue Split | Franchise cut | Franchise cut | 100% yours |
| Custom OS | No (Windows) | No | Yes (BittuOS) |
| OTA Updates | No | No | Yes + rollback |
| Express Lane | No | No | Yes |
| ERP Integration | No | No | Yes (Phase 2) |
| Telegram Bot | No | No | Yes |
| Open Source | No | No | Yes |
| DPDP Compliant | Unknown | Unknown | Yes |